# Auto Tap V1.0 Instructions

Written By: Corwin Perren  (www.caperren.com)

# Running a Tap Test

- Turn the power switch for the unit on.

- Wait for the home screen to be displayed.

- Hit the green start button to enter the settle period.

- Wait for the timer to elapse or hit the stop button to skip the settle period.

- Once on the main test timer screen, wait for the timer to elapse or hit the stop button to end the cycle early if desired.

- Upon test end or stop button pressed, you will be returned to the home screen.

# Adjusting Settings

- Turn the power switch for the unit on.

- Press the button on the rotary knob to enter the settings menu.

- Turn the knob clockwise or counterclockwise until the desired setting is shown.

- Press the button on the rotary knob to enter the value adjust screen.

- Rotate the rotary knob until the desired value is displayed.

- Press the button on the rotary knob to return to the settings menu.

- Repeat settings 3 through 6 for all remaining settings you need to change.

- Follow directions below to save settings.

## Save Settings Temporarily (Settings are lost through reboots)

- While still on the settings menu, press the stop button.

- You will be returned to the home screen and can run a test like normal

## Save Settings Permanently (Settings persist through reboots)

- While still on the settings menu, press the start button.

- The message, “Saving Settings, Please Wait” will be displayed.

- You will be returned to the home screen and can run a test like normal.

# Restoring Default Settings

- If the unit is on, turn it off.

- Hold the stop button.

- While holding the stop button, turn the unit power back on.

- Continue holding the stop button until “Restoring Settings” is displayed on the screen.

- You will be returned to the home screen and settings will now be default.

# Description of Cycle Settings (All units are in seconds)

- Settle Duration  -- Duration of the settle period

- Tap Period  -- Time between taps

- Test Duration  -- Duration of the main testing period